| Hydraulics Data |  |
| --- | --- |
| Drawing Name: | Sample Soft Hydraulic RainCad.dwg |
| Runset: | 1-2 |
| Date Generated: | 3/1/2026 12:22 AM |

| Show Data Table | Close Data Table |
| --- | --- |

|  | Water Source 1 |
| --- | --- |

| Description: | New Water Source |
| --- | --- |
| Type: | Pump |
| Pump Pressure: | 0 (bars) |

|  | Mean System |
| --- | --- |

| Static Pressure: | 0 (bars) |
| --- | --- |
| Service Line Loss: | 0 (bars) |
| Meter Loss: | 0 (bars) |
| Backflow Loss: | 0 (bars) |
| Master Valve Loss: | 0 (bars) |
| Master Filter Loss: | 0 (bars) |

|  | Mainline Pipe Runs |
| --- | --- |

| Nodes | Description | Length (m) | Flow (m^3/h) | Velocity (m/s) | Loss (bars) |
| --- | --- | --- | --- | --- | --- |
| 1>3 | 75mm | 48.352 | 14.42 | 1.109 | 0.085 |

|  | Zone: 1-2 @ 14.42 (m^3/h) |
| --- | --- |

| Valve Description: | Rain Bird 150PESB-R |
| --- | --- |
| Mainline Loss: | 0.085 (bars) |
| Valve Loss: | 0.227 (bars) |
| Filter Loss: | 0 (bars) |
| Lateral Loss: | 0.1 (bars) |
| Elevation: | 0 (bars) |
| Sprinkler Requirement: | 2.1 (bars) |
| Total Design Pressure: | 2.512 (bars) |
| Residual Pressure: | -2.512 (bars) |

|  | Zone Lateral Pipe |
| --- | --- |

| Nodes | Pipe Size | Length (m) | Flow (m^3/h) | Velocity (m/s) | Loss (bars) |
| --- | --- | --- | --- | --- | --- |
| 1-2 | 50mm | 1.03 | 5.13 | 0.888 | 0.002 |
| 3-4 | 32mm | 1.809 | 2.36 | 1.035 | 0.008 |
| 5-3 | 32mm | 1.245 | 2.41 | 1.057 | 0.006 |
| 6-7 | 50mm | 1.247 | 4.78 | 0.827 | 0.002 |
| 8-6 | 50mm | 1.193 | 4.83 | 0.836 | 0.002 |
| 9-8 | 50mm | 1.149 | 4.88 | 0.845 | 0.002 |
| 10-11 | 50mm | 0.4 | 5.81 | 1.006 | 0.001 |
| 12-13 | 75mm | 0.388 | 14.42 | 1.109 | 0.001 |
| 7-14 | 50mm | 1.705 | 4.73 | 0.819 | 0.003 |
| 1-9 | 50mm | 1.686 | 4.93 | 0.853 | 0.003 |
| 15-5 | 32mm | 1.339 | 2.46 | 1.079 | 0.006 |
| 16-15 | 32mm | 1.283 | 2.51 | 1.101 | 0.006 |
| 11-16 | 32mm | 1.868 | 2.56 | 1.123 | 0.009 |
| 17-18 | 32mm | 4.452 | 0.3 | 0.132 | 0 |
| 19-17 | 32mm | 1.865 | 0.72 | 0.316 | 0.001 |
| 20-21 | 32mm | 1.044 | 0.61 | 0.267 | 0 |
| 22-23 | 32mm | 1.24 | 1.21 | 0.531 | 0.002 |
| 24-22 | 32mm | 0.939 | 1.21 | 0.531 | 0.001 |
| 25-24 | 32mm | 1.052 | 1.01 | 0.443 | 0.001 |
| 26-25 | 32mm | 0.808 | 1.01 | 0.443 | 0.001 |
| 27-26 | 32mm | 0.99 | 0.81 | 0.355 | 0.001 |
| 28-27 | 32mm | 0.832 | 0.81 | 0.355 | 0 |
| 29-28 | 32mm | 1.025 | 0.61 | 0.267 | 0 |
| 30-29 | 32mm | 0.967 | 0.61 | 0.267 | 0 |
| 31-30 | 32mm | 1.168 | 0.3 | 0.132 | 0 |
| 32-31 | 32mm | 1.067 | 0.3 | 0.132 | 0 |
| 33-32 | 32mm | 1.334 | 0.1 | 0.044 | 0 |
| 34-33 | 32mm | 1.24 | 0.1 | 0.044 | 0 |
| 35-19 | 32mm | 1.993 | 0.72 | 0.316 | 0.001 |
| 36-35 | 32mm | 0.579 | 1.12 | 0.491 | 0.001 |
| 37-36 | 50mm | 2.634 | 4.22 | 0.731 | 0.003 |
| 14-37 | 50mm | 2.34 | 4.53 | 0.784 | 0.003 |
| 38-39 | 32mm | 1.51 | 3.27 | 1.434 | 0.012 |
| 40-38 | 32mm | 2.756 | 3.06 | 1.342 | 0.019 |
| 41-40 | 32mm | 1.563 | 2.75 | 1.206 | 0.009 |
| 42-41 | 32mm | 1.539 | 2.75 | 1.206 | 0.009 |
| 43-42 | 32mm | 1.242 | 2.35 | 1.03 | 0.005 |
| 44-45 | 32mm | 1.717 | 1.81 | 0.794 | 0.004 |
| 46-44 | 32mm | 1.568 | 1.41 | 0.618 | 0.003 |
| 23-46 | 32mm | 1.111 | 1.41 | 0.618 | 0.002 |
| 47-48 | 32mm | 0.895 | 0.21 | 0.092 | 0 |
| 49-50 | 32mm | 4.035 | 2.03 | 0.89 | 0.013 |
| 51-49 | 32mm | 3.854 | 1.61 | 0.706 | 0.008 |
| 52-51 | 32mm | 2.889 | 1.34 | 0.588 | 0.004 |
| 53-52 | 32mm | 2.029 | 1.13 | 0.496 | 0.002 |
| 54-53 | 32mm | 1.908 | 1 | 0.439 | 0.002 |
| 55-54 | 32mm | 1.883 | 0.87 | 0.382 | 0.001 |
| 56-55 | 32mm | 1.892 | 0.74 | 0.324 | 0.001 |
| 57-56 | 32mm | 2.073 | 0.61 | 0.267 | 0.001 |
| 58-50 | 32mm | 3.619 | 2.03 | 0.89 | 0.012 |
| 59-60 | 32mm | 1.636 | 0.61 | 0.267 | 0.001 |
| 61-59 | 32mm | 2.147 | 1.03 | 0.452 | 0.002 |
| 62-61 | 32mm | 2.194 | 1.03 | 0.452 | 0.002 |
| 63-62 | 32mm | 3.705 | 1.45 | 0.636 | 0.006 |
| 64-63 | 32mm | 3.361 | 1.85 | 0.811 | 0.009 |
| 4-64 | 32mm | 2.271 | 2.16 | 0.947 | 0.008 |
| 65-11 | 32mm | 1.177 | 3.05 | 1.337 | 0.008 |
| 66-65 | 32mm | 1.182 | 3.05 | 1.337 | 0.008 |
| 67-66 | 32mm | 1.401 | 2.74 | 1.202 | 0.008 |
| 68-67 | 32mm | 1.525 | 2.74 | 1.202 | 0.009 |
| 69-68 | 32mm | 1.726 | 2.43 | 1.066 | 0.008 |
| 58-69 | 32mm | 1.749 | 2.43 | 1.066 | 0.008 |
| 70-71 | 32mm | 1.867 | 0.61 | 0.267 | 0.001 |
| 72-70 | 32mm | 2.014 | 0.61 | 0.267 | 0.001 |
| 73-72 | 32mm | 0.958 | 1.45 | 0.636 | 0.002 |
| 74-75 | 32mm | 0.85 | 0.27 | 0.118 | 0 |
| 76-77 | 32mm | 1.042 | 0.54 | 0.237 | 0 |
| 78-73 | 32mm | 3.645 | 0.84 | 0.368 | 0.002 |
| 79-43 | 32mm | 3.356 | 0.54 | 0.237 | 0.001 |
| 80-81 | 32mm | 1.03 | 0.13 | 0.057 | 0 |
| 82-83 | 32mm | 0.837 | 0.13 | 0.057 | 0 |
| 84-82 | 32mm | 0.892 | 0.4 | 0.175 | 0 |
| 57-85 | 32mm | 1.079 | 0.4 | 0.175 | 0 |
| 13-2 | 63mm | 0.385 | 10.94 | 1.191 | 0.001 |
| 86-10 | 50mm | 0.378 | 5.81 | 1.006 | 0.001 |
| 87-86 | 50mm | 0.943 | 5.81 | 1.006 | 0.002 |
| 2-87 | 50mm | 0.419 | 5.81 | 1.006 | 0.001 |
| 88-20 | 32mm | 1.145 | 0.61 | 0.267 | 0 |
| 60-88 | 32mm | 1.232 | 0.61 | 0.267 | 0 |
| 48-13 | 50mm | 0.322 | 3.48 | 0.602 | 0 |
| 89-48 | 32mm | 0.328 | 3.27 | 1.434 | 0.003 |
| 39-89 | 32mm | 0.605 | 3.27 | 1.434 | 0.005 |
| 45-43 | 32mm | 0.485 | 1.81 | 0.794 | 0.001 |
| 77-73 | 32mm | 2.713 | 2.29 | 1.004 | 0.011 |
| 75-77 | 32mm | 1.826 | 2.83 | 1.241 | 0.011 |
| 90-36 | 32mm | 1.268 | 3.1 | 1.359 | 0.009 |
| 75-90 | 32mm | 1.167 | 3.1 | 1.359 | 0.008 |
| 83-80 | 32mm | 0.948 | 0.13 | 0.057 | 0 |
| 85-84 | 32mm | 1.058 | 0.4 | 0.175 | 0 |